Supplementary Information for Advancing Sustainability and Decarbonization through Circularity in Informal E-Waste Supply Chains in Emerging Economies: Evidence from Indonesia

Table S1. System Dynamics Model Equations

| Type | Variable | Equation | Unit |
| --- | --- | --- | --- |
| Stock | Collected E-waste stock | INTEG (Collection flow-Formal processing flow-Informal processing flow , Initial collected E-waste) | Ton |
| Stock | Cumulative CO2 Reduction | INTEG (Annual CO2 reduction * STEP(1, Policy start year), 0) | Kg CO2 Eq |
| Stock | E-waste stock | INTEG (E-waste generation-Collection flow-Leakage to uncontrolled disposal , Initial E-waste stock) | Ton |
| Stock | Formal recycling capacity | INTEG (Capacity addition-Capacity depreciation, Initial formal recycling capacity) | Ton/Year |
| Stock | Pollution burden | INTEG (Pollution generation-Pollution mitigation, Initial pollution burden) | Pollution index |
| Stock | Recovered Material Stock | INTEG (Recovered material flow-Secondary material use, Initial recovered material stock) | Ton |
| Stock | Social inclusion of informal workers | INTEG (Integration improvement-Integration erosion , Initial Social inclusion of informal workers) | Dmnl |
| Auxiliary/flow | Access to clean technology | Historical access to clean technology + (Scenario access to clean technology - Historical access to clean technology) * RAMP(1 / Implementation period, Policy start year , Policy start year + Implementation period) | Dmnl |
| Auxiliary/flow | Annual CO2 reduction | Virgin material displacement * Emission factor avoided virgin material - Formal processing flow * Emission factor formal recycling - Informal processing flow * Emission factor informal recycling | Kg CO2 Eq/Year |
| Auxiliary/flow | Capacity addition | Investment attractiveness*MAX(Max capacity addition, Capacity programme switch * STEP(1, Policy start year) * Formal capacity gap / Capacity adjustment time) | Ton/Year/Year |
| Auxiliary/flow | Capacity depreciation | Formal recycling capacity/Capacity lifetime | Ton/Year/Year |
| Auxiliary/flow | Collection and reverse logistics efficiency | MIN(0.95, Base collection efficiency+ 0.25 * Reverse logistics efficiency + 0.2 * Social inclusion of informal workers + 0.15 * Data transparency and traceability) | Dmnl |
| Auxiliary/flow | Collection flow | E-waste stock * Collection and reverse logistics efficiency / Collection Time | Ton/Year |
| Auxiliary/flow | Data transparency and traceability | MIN(1, 0.2 + 0.3 * Stakeholder collaboration+ 0.25 * EPR Implementation+ 0.2 * Regulatory enforcement) | Dmnl |
| Auxiliary/flow | Desired integration | MIN(1, 0.2 + 0.3 * Stakeholder collaboration+ 0.25 * Regulatory enforcement + 0.2 * Economic incentives + 0.15 * Occupational health and safety) | Dmnl |
| Auxiliary/flow | E-waste generation | Saturation level / (1 + EXP( - Logistic growth rate * (Time - Inflection year))) * (1 - Life cycle extension effect) | Ton/Year |
| Auxiliary/flow | Economic incentives | Historical economic incentives + (Scenario economic incentives - Historical economic incentives) * RAMP(1 / Implementation period, Policy start year , Policy start year + Implementation period) | Dmnl |
| Auxiliary/flow | EPR Implementation | Historical EPR implementation + (Scenario EPR Implementation - Historical EPR implementation) * RAMP(1 / Implementation period, Policy start year , Policy start year + Implementation period) | Dmnl |
| Auxiliary/flow | Formal capacity gap | MAX(0, Collected E-waste stock * Formal processing share / Processing time - Formal recycling capacity) | Ton/Year |
| Auxiliary/flow | Formal processing flow | MIN(Collected E-waste stock * Formal processing share/ Processing time ,Formal recycling capacity ) | Ton/Year |
| Auxiliary/flow | Formal processing share | MIN(0.95, Base formal share+ 0.25 * Regulatory enforcement+ 0.2 * EPR Implementation+ 0.25 * Social inclusion of informal workers + 0.15 * Access to clean technology) | Dmnl |
| Auxiliary/flow | Formal recovery efficiency | MIN(0.9, Base formal recovery efficiency+ 0.2 * Technology adoption index + 0.1 * Data transparency and traceability) | Dmnl |
| Auxiliary/flow | Informal processing flow | MAX(0,Collected E-waste stock / Processing time - Formal processing flow ) | Ton/Year |
| Auxiliary/flow | Informal recovery efficiency | MAX(0.2, Base informal recovery efficiency+ 0.1 * Social inclusion of informal workers - 0.1 * Regulatory restriction effect ) | Dmnl |
| Auxiliary/flow | Initial collected E-waste | 1.17418e+06 | Ton |
| Auxiliary/flow | Initial E-waste stock | 1.95645e+06 | Ton |
| Auxiliary/flow | Integration erosion | Social inclusion of informal workers* (1 - Stakeholder collaboration ) / Integration erosion time | 1/Year |
| Auxiliary/flow | Integration improvement | MAX ( 0 , (Desired integration-Social inclusion of informal workers )/Integration adjustment time ) | 1/Year |
| Auxiliary/flow | Investment attractiveness | MIN(1, 0.2 + 0.25 * Economic incentives+ 0.25 * Policy effectiveness + 0.2 * Stakeholder collaboration + 0.1 * Data transparency and traceability) | Dmnl |
| Auxiliary/flow | Leakage fraction | MAX(Minimum leakage fraction,Base leakage fraction* (1 - 0.5 * Regulatory enforcement)* (1 - 0.3 * Collection and reverse logistics efficiency)) | 1/Year |
| Auxiliary/flow | Leakage to uncontrolled disposal | MAX ( 0 , E-waste stock*Leakage fraction ) | Ton/Year |
| Auxiliary/flow | Life cycle extension effect | 0.25 * Lifecycle extension | Dmnl |
| Auxiliary/flow | Lifecycle extension | MIN(1, Base lifecycle extension+ 0.3 * Policy effectiveness+ 0.25 * Data transparency and traceability+ 0.2 * Social inclusion of informal workers ) | Dmnl |
| Auxiliary/flow | Occupational health and safety | MIN(1, 0.2 + 0.3 * Regulatory enforcement+ 0.25 * Social inclusion of informal workers + 0.25 * Access to clean technology) | Dmnl |
| Auxiliary/flow | Policy effectiveness | MIN(1, 0.35 * Regulatory enforcement+ 0.3 * EPR Implementation+ 0.35 * Stakeholder collaboration) | Dmnl |
| Auxiliary/flow | Pollution control practices | MIN(0.9,0.15 + 0.3 * Access to clean technology+ 0.25 * Regulatory enforcement + 0.2 * Technology adoption index ) | Dmnl |
| Auxiliary/flow | Pollution generation | Informal processing flow * Informal pollution factor+ Leakage to uncontrolled disposal * Disposal pollution factor + Formal processing flow * Formal pollution factor | Pollution index/Year |
| Auxiliary/flow | Pollution mitigation | Pollution burden*Pollution control practices/Pollution mitigation time | Pollution index/Year |
| Auxiliary/flow | Recovered material flow | Formal processing flow* Formal recovery efficiency + Informal processing flow * Informal recovery efficiency | Ton/Year |
| Auxiliary/flow | Regulatory enforcement | Historical regulatory enforcement + (Scenario regulatory enforcement - Historical regulatory enforcement) * RAMP(1 / Implementation period, Policy start year , Policy start year + Implementation period) | Dmnl |
| Auxiliary/flow | Regulatory restriction effect | Regulatory enforcement * (1 - Social inclusion of informal workers ) | Dmnl |
| Auxiliary/flow | Reverse logistics efficiency | MIN(1, 0.25 + 0.3 * Stakeholder collaboration + 0.2 * Economic incentives + 0.15 * Data transparency and traceability ) | Dmnl |
| Auxiliary/flow | Saturation level | 3.90284e+06 | Ton/Year |
| Auxiliary/flow | Secondary material use | Recovered Material Stock/Secondary material use time | Ton/Year |
| Auxiliary/flow | Stakeholder collaboration | Historical stakeholder collaboration + (Scenario stakeholder collaboration - Historical stakeholder collaboration) * RAMP(1 / Implementation period, Policy start year , Policy start year + Implementation period) | Dmnl |
| Auxiliary/flow | Technological upgrading capability | MIN(1, 0.25 + 0.3 * Economic incentives+ 0.3 * Access to clean technology + 0.15 * Data transparency and traceability) | Dmnl |
| Auxiliary/flow | Technology adoption index | MIN(1, 0.4 * Access to clean technology + 0.35 * Technological upgrading capability + 0.25 * Economic incentives) | Dmnl |
| Auxiliary/flow | Virgin material displacement | Secondary material use*Substitution ratio | Ton/Year |

Simulation control: INITIAL TIME = 2016, FINAL TIME = 2060, TIME STEP = 0.25 year (Euler), SAVEPER = 1 year. Scenario inputs are phased in linearly over 2025–2030; the cumulative net carbon benefit is accumulated from 2025.

Table S2. Parameter values, units, ranges used in the uncertainty analysis and provenance

| Parameter | Value | Unit | Uncertainty range | Provenance |
| --- | --- | --- | --- | --- |
| Base collection efficiency | 0.35 | Dmnl | – | Behavioural assumption (stylized) |
| Base formal recovery efficiency | 0.6 | Dmnl | – | Behavioural assumption (stylized) |
| Base formal share | 0.25 | Dmnl | – | Behavioural assumption (stylized) |
| Base informal recovery efficiency | 0.4 | Dmnl | – | Behavioural assumption (stylized) |
| Base leakage fraction | 0.08 | 1/Year | – | Behavioural assumption (stylized) |
| Base lifecycle extension | 0.15 | Dmnl | – | Behavioural assumption (stylized) |
| Capacity adjustment time | 10 | Year | 5–20 | Assumption: time to plan, finance, permit and build facilities |
| Capacity lifetime | 15 | Year | 10–20 | Behavioural assumption (stylized) |
| Capacity programme switch | 0 | Dmnl | – | Scenario switch (1 in ICT + CP) |
| Collection Time | 1 | Year | – | Behavioural assumption (stylized) |
| Disposal pollution factor | 0.012 | Pollution index/Ton | – | Behavioural assumption (stylized) |
| Emission factor avoided virgin material | 2500 | Kg CO2 Eq/Ton | 1,500–3,500 | Calibrated: implied 1.5–1.8 t CO2e avoided per t formally processed; conservative vs ~3.8 t/t in Baldé et al. (2024) |
| Emission factor formal recycling | 500 | Kg CO2 Eq/Ton | 300–700 | Calibrated: implied net saving 1.0–1.3 t CO2e/t, cf. −0.85 t/t (Clarke et al., 2019) and −0.23 to −1.35 t/t (Turner et al., 2015) |
| Emission factor informal recycling | 900 | Kg CO2 Eq/Ton | 600–1,200 | Higher than formal due to open burning; ~1.6 t CO2 per t plastics burned (IPCC, 2006 defaults) |
| Formal pollution factor | 0.002 | Pollution index/Ton | – | Behavioural assumption (stylized) |
| Historical access to clean technology | 0.25 | Dmnl | – | Policy setting 2016–2024 (= BAU) |
| Historical economic incentives | 0.25 | Dmnl | – | Policy setting 2016–2024 (= BAU) |
| Historical EPR implementation | 0.25 | Dmnl | – | Policy setting 2016–2024 (= BAU) |
| Historical regulatory enforcement | 0.3 | Dmnl | – | Policy setting 2016–2024 (= BAU) |
| Historical stakeholder collaboration | 0.3 | Dmnl | – | Policy setting 2016–2024 (= BAU) |
| Implementation period | 5 | Year | – | Scenario design (phase-in 2025–2030) |
| Inflection year | 2020.51 | Year | – | Fitted (see above) |
| Informal pollution factor | 0.01 | Pollution index/Ton | – | Behavioural assumption (stylized) |
| Initial formal recycling capacity | 84433.5 | Ton/Year | – | 2016 equilibrium under historical policy (spin-up), consistent with calibration above |
| Initial pollution burden | 169333 | Pollution index | – | 2016 equilibrium (spin-up) |
| Initial recovered material stock | 516160 | Ton | – | 2016 equilibrium (spin-up) |
| Initial Social inclusion of informal workers | 0.3770 | Dmnl | – | 2016 equilibrium (spin-up) |
| Integration adjustment time | 4 | Year | – | Behavioural assumption (stylized) |
| Integration erosion time | 10 | Year | – | Behavioural assumption (stylized) |
| Logistic growth rate | 0.1211 | 1/Year | – | Fitted (see above) |
| Max capacity addition | 13000 | Ton/Year/Year | 6,500–26,000 | Calibrated: formal recycling ≈5% of generation in 2020 (Ministry of Environment and Forestry, 2024) |
| Minimum leakage fraction | 0.02 | 1/Year | – | Model constant |
| Policy start year | 2025 | Year | – | Scenario design |
| Pollution mitigation time | 5 | Year | – | Behavioural assumption (stylized) |
| Processing time | 1 | Year | – | Behavioural assumption (stylized) |
| Secondary material use time | 1 | Year | – | Behavioural assumption (stylized) |
| Substitution ratio | 0.85 | Dmnl | 0.60–0.95 | Within 0.49–1.0 range of substitutability coefficients (Rigamonti et al., 2020) |

Scenario constants (Scenario regulatory enforcement, Scenario EPR Implementation, Scenario stakeholder collaboration, Scenario access to clean technology, Scenario economic incentives) take the values in Table 4 of the manuscript; the file is saved with BAU values and Capacity programme switch = 0.

Table S3. Callibration and behaviour reproduction

| Indicator | Observed / reference | Simulated | Difference |
| --- | --- | --- | --- |
| E-waste generation 2016 (Mt) | 1.274 (GEM 2017; Baldé et al., 2017) | 1.283 | +0.7% |
| E-waste generation 2019 (Mt) | 1.618 (GEM 2020; Forti et al., 2020) | 1.590 | −1.7% |
| E-waste generation 2022 (Mt) | 1.886 (GEM 2024; Baldé et al., 2024) | 1.907 | +1.1% |
| Mean absolute percentage error (GEM points) | – | – | 1.2% |
| E-waste generation 2040 (Mt) | 3.2 (projection; Mairizal et al., 2021) | 3.197 | −0.1% |
| E-waste generation 2021 (Mt), independent check | 2.0 (Mairizal et al., 2021; different method) | 1.801 | −10% |
| Formal recycling rate 2020 (% of generation) | ≤5% (Ministry of Environment and Forestry, 2024) | 5.0% | calibration target |
| Informal share of processing 2020 | ≈90% (Ministry of Environment and Forestry, 2024) | 94% | consistent |
| Leakage to uncontrolled disposal 2020 (% of generation) | – | 7.7% | – |
| Formal recycling capacity 2020 (kt/yr) | – | 85 | – |

Table S4. Model Testing results

| Test | Result | Assessment |
| --- | --- | --- |
| Dimensional consistency | Units assigned to all variables; Minimum leakage fraction added as named constant | Pass |
| Extreme condition: all five inputs = 0 | All stocks ≥ 0; inclusion index 0.17–0.38; formal share 1.5% in 2060; cumulative carbon −0.41 Mt CO₂e | Pass |
| Extreme condition: all five inputs = 1 | All stocks ≥ 0; inclusion index ≤ 1.00; cumulative carbon 16.23 Mt CO₂e; formal share 6.2% | Pass |
| Extreme condition: all inputs = 1 + capacity programme | Formal share 55.7%; cumulative carbon 59.43 Mt CO₂e | Pass |
| Extreme condition: e-waste generation ×0.01 (ICT) | Stocks drain; formal share reaches the 95% cap; pollution burden → 0.5 thousand | Pass |
| Extreme condition: maximum capacity addition = 0 (ICT) | Formal capacity depreciates to 0; formal share → 0 | Pass |
| Extreme condition: avoided-emission factor = 0 (ICT) | Net carbon balance negative (−82.06 Mt CO₂e) as expected | Pass |
| Integration error (DT 1 → 0.5 → 0.25 → 0.125) | Cumulative carbon 2060 (ICT + CP): 50.78 / 51.28 / 51.53 / 51.66 Mt CO₂e; change 0.25→0.125 ≤ 0.24% in all scenarios | Pass (DT 0.25 adopted) |
| Implementation check (Vensim file run with PySD vs Python code) | Cumulative carbon 2060 identical to 3 decimals in all five scenarios; formal processing within 1.2×10⁻⁶ relative difference | Pass |
| Behaviour reproduction | See Table S3 (MAPE 1.2% for generation; formal recycling rate 5.0%) | Pass (limited data) |

Table S5. One-way sensitivity results (values in 2060)

| Scenario | Parameter | Low / high value | Cumulative net carbon benefit (Mt CO₂e) | Formal share (%) | Pollution burden (thousand) |
| --- | --- | --- | --- | --- | --- |
| BAU | EF avoided virgin material (kg CO₂e/t) | 1,500 / 3,500 | -33.88 / 35.39 | 2.7 / 2.7 | 426 / 426 |
| BAU | EF formal recycling (kg CO₂e/t) | 300 / 700 | 1.35 / 0.16 | 2.7 / 2.7 | 426 / 426 |
| BAU | EF informal recycling (kg CO₂e/t) | 600 / 1,200 | 28.87 / -27.37 | 2.7 / 2.7 | 426 / 426 |
| BAU | Substitution ratio (–) | 0.60 / 0.95 | -24.72 / 10.94 | 2.7 / 2.7 | 426 / 426 |
| BAU | Max capacity addition (t/yr²) | 6,500 / 26,000 | -0.75 / 3.75 | 1.3 / 5.4 | 431 / 417 |
| BAU | Saturation level (t/yr) | 2,927,129 / 4,878,549 | 1.31 / 0.19 | 3.6 / 2.1 | 317 / 535 |
| BAU | Capacity lifetime (yr) | 10.00 / 20.00 | -0.24 / 1.73 | 1.8 / 3.6 | 429 / 423 |
| BAU | Capacity adjustment time (yr) | 5.00 / 20.00 | 0.75 / 0.75 | 2.7 / 2.7 | 426 / 426 |
| ICT | EF avoided virgin material (kg CO₂e/t) | 1,500 / 3,500 | -25.09 / 50.87 | 5.3 / 5.3 | 187 / 187 |
| ICT | EF formal recycling (kg CO₂e/t) | 300 / 700 | 13.80 / 11.98 | 5.3 / 5.3 | 187 / 187 |
| ICT | EF informal recycling (kg CO₂e/t) | 600 / 1,200 | 39.49 / -13.70 | 5.3 / 5.3 | 187 / 187 |
| ICT | Substitution ratio (–) | 0.60 / 0.95 | -15.03 / 24.06 | 5.3 / 5.3 | 187 / 187 |
| ICT | Max capacity addition (t/yr²) | 6,500 / 26,000 | 10.21 / 18.25 | 2.7 / 10.6 | 191 / 180 |
| ICT | Saturation level (t/yr) | 2,927,129 / 4,878,549 | 11.01 / 14.78 | 7.1 / 4.3 | 139 / 236 |
| ICT | Capacity lifetime (yr) | 10.00 / 20.00 | 11.35 / 14.30 | 3.7 / 6.8 | 190 / 185 |
| ICT | Capacity adjustment time (yr) | 5.00 / 20.00 | 12.89 / 12.89 | 5.3 / 5.3 | 187 / 187 |
| ICT + CP | EF avoided virgin material (kg CO₂e/t) | 1,500 / 3,500 | 3.43 / 99.64 | 52.2 / 52.2 | 121 / 121 |
| ICT + CP | EF formal recycling (kg CO₂e/t) | 300 / 700 | 59.10 / 43.96 | 52.2 / 52.2 | 121 / 121 |
| ICT + CP | EF informal recycling (kg CO₂e/t) | 600 / 1,200 | 68.13 / 34.93 | 52.2 / 52.2 | 121 / 121 |
| ICT + CP | Substitution ratio (–) | 0.60 / 0.95 | 16.16 / 65.68 | 52.2 / 52.2 | 121 / 121 |
| ICT + CP | Max capacity addition (t/yr²) | 6,500 / 26,000 | 51.15 / 52.30 | 52.2 / 52.2 | 121 / 120 |
| ICT + CP | Saturation level (t/yr) | 2,927,129 / 4,878,549 | 38.84 / 64.22 | 52.2 / 52.2 | 90 / 151 |
| ICT + CP | Capacity lifetime (yr) | 10.00 / 20.00 | 45.16 / 55.65 | 43.2 / 58.1 | 133 / 113 |
| ICT + CP | Capacity adjustment time (yr) | 5.00 / 20.00 | 68.94 / 35.66 | 67.8 / 35.2 | 96 / 146 |

Baseline values (2060): BAU 0.75 Mt CO₂e, 2.7%, 426; ICT 12.89 Mt CO₂e, 5.3%, 187; ICT + CP 51.53 Mt CO₂e, 52.2%, 121.

Table S6. Key System Dynamics results for Indonesia by scenario (2025–2060; time step 0.25 year)

| Indicator | BAU | Regulatory Push | Technology Push | ICT | ICT + CP |
| --- | --- | --- | --- | --- | --- |
| Cumulative net carbon benefit, 2035 (Mt CO₂e) | −0.10 | 0.16 | 0.26 | 0.73 | 4.39 |
| Cumulative net carbon benefit, 2045 (Mt CO₂e) | 0.10 | 2.28 | 1.77 | 4.91 | 20.31 |
| Cumulative net carbon benefit, 2060 (Mt CO₂e) | 0.75 | 6.53 | 4.77 | 12.89 | 51.53 |
| Year cumulative balance turns positive | 2042 | 2034 | 2033 | 2033 | 2029 |
| Formal processing share, 2030 (%) | 3.8 | 4.0 | 3.9 | 4.1 | 18.0 |
| Formal processing share, 2045 (%) | 2.8 | 4.2 | 3.6 | 5.0 | 47.3 |
| Formal processing share, 2060 (%) | 2.7 | 4.4 | 3.6 | 5.3 | 52.2 |
| Formal recycling capacity, 2060 (kt/yr) | 85 | 132 | 114 | 158 | 1,555 |
| Formal / informal processing, 2060 (Mt/yr) | 0.08 / 3.08 | 0.13 / 2.90 | 0.11 / 3.02 | 0.16 / 2.82 | 1.56 / 1.43 |
| Recovered material flow, 2060 (Mt/yr) | 1.35 | 1.40 | 1.41 | 1.50 | 2.01 |
| Social inclusion index, 2060 (–) | 0.377 | 0.706 | 0.544 | 0.907 | 0.907 |
| Pollution burden, 2045 (index, thousand) | 348 | 249 | 227 | 174 | 127 |
| Pollution burden, 2060 (index, thousand) | 426 | 279 | 253 | 187 | 121 |
| E-waste generation, 2060 (Mt/yr) | 3.47 | 3.20 | 3.38 | 3.11 | 3.11 |

Table S7. Single-lever experiments: one input raised from its BAU value to its ICT value (or the capacity programme activated), others held at BAU values

| Experiment | Cumulative net carbon benefit, 2060 (Mt CO₂e) | Change vs BAU (Mt CO₂e) | Formal share, 2045 (%) | Social inclusion, 2060 | Pollution burden, 2060 (thousand) |
| --- | --- | --- | --- | --- | --- |
| BAU | 0.75 | – | 2.8 | 0.377 | 426 |
| BAU + regulatory enforcement (0.85) | −1.19 | −1.94 | 3.1 | 0.507 | 313 |
| BAU + EPR implementation (0.80) | 1.11 | +0.36 | 3.1 | 0.377 | 415 |
| BAU + stakeholder collaboration (0.85) | 6.62 | +5.87 | 3.7 | 0.619 | 406 |
| BAU + access to clean technology (0.85) | 1.49 | +0.74 | 2.8 | 0.395 | 275 |
| BAU + economic incentives (0.80) | 3.43 | +2.68 | 3.4 | 0.465 | 389 |
| Sum of the five enabler effects | – | +7.71 | – | – | – |
| ICT (five enablers) | 12.89 | +12.14 | 5.0 | 0.907 | 187 |
| BAU + capacity programme | 12.26 | +11.51 | 16.5 | 0.377 | 383 |
| ICT + capacity programme | 51.53 | +50.78 | 47.3 | 0.907 | 121 |

Note. The ICT effect (+12.14 Mt CO₂e) exceeds the sum of the five enabler effects by 58%; the ICT + CP effect exceeds the sum of the ICT and capacity-programme effects (+23.65 Mt CO₂e) by a factor of 2.1.

Table S8. Monte Carlo results (2,000 runs per scenario; eight parameters sampled uniformly over the ranges in Section 3.7)

| Scenario | Median cumulative benefit, 2060 (Mt CO₂e) | 5th–95th percentile (Mt CO₂e) | P(cumulative > 0), 2045 | P(cumulative > 0), 2060 | Median pollution burden, 2060 (thousand) |
| --- | --- | --- | --- | --- | --- |
| BAU | −7.63 | −50.29 to 41.53 | 0.39 | 0.39 | 422 |
| Regulatory Push | −1.98 | −45.51 to 48.11 | 0.44 | 0.47 | 275 |
| Technology Push | −3.80 | −47.77 to 46.74 | 0.43 | 0.45 | 250 |
| ICT | 4.04 | −40.14 to 56.22 | 0.51 | 0.55 | 184 |
| ICT + CP | 34.79 | −13.34 to 96.72 | 0.78 | 0.87 | 126 |

Monte Carlo setting

2,000 runs per scenario; eight parameters sampled independently from uniform distributions over the ranges in Table S2; common random numbers across scenarios (seed 2026); initial stocks recomputed (spin-up) for each parameter set. Spearman rank correlations with cumulative net carbon benefit in 2060 (ICT + CP / ICT / BAU): avoided virgin EF 0.77 / 0.73 / 0.69; substitution ratio 0.44 / 0.42 / 0.40; informal EF −0.34 / −0.56 / −0.61; capacity adjustment time −0.24 / −0.01 / −0.01; formal EF −0.11 / −0.01 / 0.00; saturation level 0.11 / −0.03 / −0.07; capacity lifetime 0.10 / 0.04 / 0.03; maximum capacity addition 0.01 / 0.08 / 0.05.

Dynamic modeling and Simulation

For the simulation modeling and result, the comprehensive file is availible in the folliwing link: https://github.com/Alfarisi006/E-waste-Supply-Chain